# Exam preperations PFAS – Quesiton in lecture slides

# Rad to degree and degree to radians

To get gradient orientation from Gx and Gy, do:
atan(Gy, Gx)

Theme, Question

# Lecture 1 – Image Processing

What is Image Processing?

Image processing is:

the operations we perform on an image to change or enhance it and make it suitable for further analysis, so that useful information can be highlighted or get extracted from it.

Linear Filtering

What are Linear Filters in Image Processing?
Linear filters are operations that modify an image by applying a weighted sum of pixel values using a kernel (convolution).

What are they used for?
They are used for noise reduction, smoothing, sharpening, edge detection, and general enhancement or suppression of spatial details.

Cross-correlation
What it is:
You slide the kernel over the image and compute
S(m,n) = sum over i,j of w(i,j) * f(m + i, n + j)
The kernel is used exactly as written. No flipping.

When it’s used:
In all practical vision code (OpenCV, PyTorch, TensorFlow) and in CNNs.

Convolution
What it is:
Same operation, but the kernel is flipped 180 degrees first. The formula is
S(m,n) = sum over i,j of w(i,j) * f(m − i, n − j)
The minus signs mean the kernel is reversed in both directions.

When it’s used:
In signal processing theory, Fourier analysis, and mathematically clean derivations.

Correlation = Convolution with the kernel flipped 180 degrees.
And therefore:
Convolution = Correlation with the kernel flipped 180 degrees.

How they relate:
Convolution = cross-correlation with a flipped kernel.
If you take a correlation kernel and rotate it 180 degrees, you have the convolution kernel.

When to use which:
Use cross-correlation for all practical filtering and CNNs.
Use convolution when doing theoretical signal-processing math.

## Filters

Filtering

Form a new image whose pixels are a combination of the original pixels

Why filtering?

To get useful information from images

E.g., extract edges or contours (to understand shape)

To enhance the image

E.g., to blur to remove noise

E.g., to sharpen to “enhance image” a la CSI

Linear filters
(These apply a weighted sum → output is a linear combination of input values.)

• Box filter (mean filter)
• Gaussian blur
• Sobel filter
• Prewitt filter
• Laplacian filter
• Sharpening filter
• Emboss filter
• Motion blur filter
• Any convolution/cross-correlation filter with fixed weights

Non-linear filters
(These do not compute a weighted sum → the operation breaks linearity.)

• Median filter
• Max filter
• Min filter
• Bilateral filter
• Kuwahara filter
• Morphological operations: erosion, dilation, opening, closing
• Non-local means denoising
• Adaptive filters (where weights depend on pixel content)

- Rectification (ReLU)

Difference (very short)
Linear filters: output = weighted sum of input pixels, so they obey linearity (additivity + scaling).
Non-linear filters: output is chosen by rules like median, max, comparisons, or data-dependent weights, so linearity is broken.

Morphology

• Erosion: used to shrink objects and remove small foreground noise.
• Dilation: used to grow objects and fill small gaps.
• Opening: used to remove small objects while preserving larger shapes.
• Closing: used to fill small holes and connect nearby objects.

# Lecture 2 – Edge detection, Fitting, Image features

CANNY EDGE DETECTION

- Smooth image with Gaussian blur to suppress noise;

- Compute (g_x), (g_y), gradient magnitude, and orientation using Sobel;

- Apply non-maximum suppression to keep only local maxima along gradient direction;

- Use double thresholds to label strong, weak, and non-edge pixels;

- Perform hysteresis: keep weak pixels only if connected to strong ones;
→ Output final clean, thin, connected edges.

Edge Tracking by Hysteresis

“Hysteresis” means that weak edges survive only if they are connected to strong edges.

Why?

True edges often become weak in some places (due to shadows or texture), but noise can also look like weak edges.
This step ensures:

- Weak edges linked to strong edges = kept

- Isolated weak edges = removed

This creates clean, connected edge contours.

# RANSAC (RANdom SAmple Consensus) – Expanded Explanation

RANSAC is a robust model-fitting algorithm designed to estimate parameters (e.g., a line, plane, homography) even when the data contains many outliers.

### Algorithm

- Random sampling:
Randomly select the minimum number of points needed to fit your model
(e.g., 2 points for a line, 3 for a plane, etc.).

- Model estimation:
Compute a model hypothesis using only these sampled points.

- Inlier scoring (consensus check):
For every remaining point, compute its error distance to the model.
If the error is below a threshold δ, it is counted as an inlier.
The quality of the model = number (or ratio) of inliers.

- Repeat:
Perform many random trials to find the model that gets the largest consensus set.
More iterations → higher chance of selecting an all-inlier sample.

- Refit (optional but common):
Once the best inlier set is found, refit the model using all inliers for higher accuracy.

### Key Concepts

- Inlier: A point consistent with the model (distance < δ).

- Outlier: A point that does not fit the model (distance ≥ δ).

- δ (inlier threshold): Controls tolerance: small δ = strict, large δ = tolerant.

- Consensus set: The set of all inliers for a given model hypothesis.

### Why RANSAC works

Unlike least squares, which breaks down when outliers exist, RANSAC isolates outliers by repeatedly hypothesizing models from tiny random subsets and only keeping the model that is supported by the majority of data.

### Practical example

When detecting lines:

- Each iteration samples 2 points → fits a line.

- All other points are checked: are they close enough to this line (within δ)?

- The line with the most inliers is selected as the final robust estimate.

## Harris Corner detection

## KD-tree function in OpenCV – FLANN

## Questions from slide:

- Good feature?
Distinctive, repeatable, high-contrast (not edge or flat).

- Scale & rotation invariance?
Scale-space (DoG) + dominant orientation.

- Detect points?
Find stable extrema or corners (e.g., SIFT/DoG, Harris, FAST).

- Describe points?
Use gradients around the keypoint → build a normalized descriptor.

- Match features?
Nearest-neighbor search + Lowe’s ratio test (KD-tree/FLANN for speed).

- Use features for?
Matching, stitching, SLAM, 3D reconstruction, tracking, recognition.

## Main point using Difference of Gaussians (DoF)

The Difference of Gaussians (DoG) is a feature-detection method where you blur the image twice with two different Gaussian scales (one slightly more blurred than the other) and then subtract the two results.

This subtraction removes the low-frequency content and keeps the changes in intensity, which makes edges and keypoint structures stand out.

In other words:
• Gaussian blur smooths the image.
• Two different blurs capture different levels of detail.
• Their difference highlights where the image changes — edges, corners, textures.

This is why DoG is used in SIFT: it efficiently finds keypoints across multiple scales.

## What is SIFT:

Scale-Invariant Feature Transform (SIFT) is a method for detecting distinctive image keypoints and describing them in a way that is invariant to scale, rotation, and moderate viewpoint changes.

- Scale-space construction:
The image is repeatedly blurred with Gaussians at increasing scales within each octave, and each octave is downsampled by a factor of two. The Difference-of-Gaussians (DoG) is computed by subtracting adjacent Gaussian images. Keypoint candidates are detected as local extrema (maxima or minima) in the 3D DoG volume (x, y, scale).

- Keypoint refinement:
For each candidate, a quadratic fit is used to refine its location and scale. Points with very low contrast or that lie along edges (unstable keypoints) are discarded.

- Orientation assignment:
For the remaining keypoints, gradients are computed in a neighborhood around the keypoint in the Gaussian-blurred image at the correct scale. Each gradient contributes (with Gaussian weighting) to a 36-bin orientation histogram. The highest peak gives the keypoint’s orientation. If other peaks are within 80% of the maximum, additional keypoints are created with those orientations.

- Descriptor construction:
Around each oriented keypoint, a rotated 16×16 patch is divided into 4×4 subregions. For every subregion, an 8-bin orientation histogram of gradient directions is computed. These 4×4×8 = 128 values form the SIFT descriptor, which is normalized to reduce sensitivity to lighting and contrast.

SIFT outputs a set of keypoints (location, scale, orientation) and 128-dimensional descriptors that can be matched robustly across images.

# Lecture 3 – Stereo Vision

## What is the fundamental matrix

The fundamental matrix is a 3×3 matrix that encodes the epipolar geometry between two uncalibrated camera views, mapping a point in one image to its corresponding epipolar line in the other image so that all true correspondences satisfy x'TFx=0.

## FROM QUIZ:

The fundamental matrix projects a point in the right image frame to a point in the left image.

ESSENTIAL MARIX
The essential matrix includes the pose

The essential matrix is defined as:

E=[t]×R

where:

- Ris the rotation matrix describing how the second camera is oriented relative to the first.

- tis the translation vector describing how the second camera is positioned relative to the first.

- t]×is the skew-symmetric matrix that encodes the direction of translation.

- why skew symmetric? Because its only purpose is to represent the cross product as a 
matrix operation, and the cross product can only be expressed using a skew-symmetric matrix.

EPIPOLE FROM EPILINES

Short, exam-ready explanation:

All epipolar lines in an image intersect at the epipole.
So if you can see several epipolar lines, extend them (visually or mathematically) until they meet — their point of intersection is the epipole.

### How to deduce the epipole:

- Take two or more epipolar lines.

- Extend them in both directions.

- The point where all of them converge is the epipole.

- If they only become parallel inside the image, their intersection lies outside the image.

### One-sentence summary:

The epipole is the intersection point of all epipolar lines, whether that point lies inside or outside the image.

The epipole is important because it is the point where all epipolar lines meet, defining the geometry between two camera views and constraining where a matching point must lie in the other image.

FIND EPIPOLAR LINE FROM IMAGES LOOKING

To find the epipolar line of a point in one image, take its pixel coordinates xand multiply by the fundamental matrix:

l'=Fx

This gives the epipolar line in the other image.

If you’re doing it visually without F:

- Identify the epipole (where epipolar lines converge).

- Draw the line passing through the epipole and the corresponding projection ray direction.

### One-sentence summary:

Use l'=Fxor, visually, draw the line through the epipole and the point’s projection directio

## pros and cons for Sparse Stereo Correspondence:

# Pros

- Computationally fast (only match a small set of feature points).

- Robust to textureless regions (feature detectors select only reliable points).

- More accurate matches per point (SIFT/SURF/Harris give distinctive descriptors).

- Works well for wide-baseline stereo (features are viewpoint-invariant).

- Lower memory and processing requirements than dense stereo.

# Cons

- Produces very sparse depth maps (no depth between feature points).

- Fails in low-texture or uniform areas (few/no features to match).

- Not suitable for dense 3D reconstruction (too few correspondences).

- Quality depends heavily on feature detector performance.

- Does not capture fine-grained geometry (e.g., smooth surfaces, edges).

## Stereo DENSE correspondence metrics – Local / area-based methods of stereo matching:

sum of absolute differences (SAD): Measures how similar two patches are by summing the absolute brightness differences pixel-by-pixel. (element wise)
sum of squared differences (SSD): Measures similarity by summing squared brightness differences, penalizing larger mismatches more strongly. (element wise)
Normalized Cross-Correlation (NCC): Measures patch similarity by correlating intensities while normalizing for brightness and contrast differences.

## SSD - Small vs big windows

Pros: Preserve fine detail and handle depth discontinuities better.
Cons: More sensitive to noise and textureless regions, leading to unstable matches.

### Big windows

Pros: More robust to noise and low-texture areas due to stronger averaging.
Cons: Blur over depth boundaries and lose fine details, causing smearing in disparity.

## Local Methods for Dense Stereo Matching (Area-Based)

Local (area-based) stereo methods compute disparity for every pixel by comparing small image patches between the left and right images. Instead of relying on individual pixels (which are noisy), they assume that within a small neighborhood, intensity patterns remain consistent between views.

Core idea

For each pixel in the left image:

- Take a small window around it (for example 5×5 or 9×9).

- Slide this window horizontally along the corresponding epipolar line in the right image.

- Measure how similar the two patches are using a matching cost (e.g., SSD or NCC).

- The shift with the best match gives the disparity at that pixel.

This is repeated for every pixel, producing a dense disparity map.

Common matching costs

- SSD (Sum of Squared Differences): Measures raw intensity differences. Simple and fast.

- SAD (Sum of Absolute Differences): Less sensitive to large errors than SSD.

- NCC (Normalized Cross-Correlation): More robust to illumination changes.

- ZNCC (Zero-mean NCC): Removes local brightness offsets.

Role of the window size

- Small window:

  - Better at preserving details and edges

  - Worse performance in textureless regions

- Large window:

  - Smoother disparities

  - Loses detail and can cross depth boundaries (bleeding effect)

Window size is a trade-off between detail and robustness.

Strengths of local area-based methods

- Very fast

- Easy to parallelize (GPU-friendly)

- Work well on textured surfaces

- Good for real-time systems (robots, AR, etc.)

Weaknesses

- Struggle in textureless regions (e.g., white walls)

- Sensitive to lighting differences

- Tend to blur depth at object boundaries

- Cannot enforce global smoothness constraints (each pixel is solved independently)

Examples of local methods

- Traditional block matching (as used in early stereo systems)

- OpenCV’s StereoBM (block matching algorithm)

- OpenCV’s StereoSGBM uses block cost but adds semi-global optimization (hybrid)

One-sentence summary for your exam sheet

Local dense stereo methods estimate disparity by comparing small patches between left and right images using a similarity metric; they are fast and simple but limited in textureless areas and depth edges.

SGBM (Semi-Global Block Matching) avoids the classic window-size trade-off by using a small matching window (to preserve detail) but enforcing smoothness penalties across many scanline directions. Instead of relying on a large window to stabilize matching, SGBM adds a global regularization term that encourages neighboring disparities to be similar unless there's an edge. This gives the robustness of large windows in low-texture areas while still keeping sharp details at boundaries, effectively combining local matching with semi-global optimization so you don’t need to choose between “big and smooth” or “small and detailed.”

## Global stereo matching

Global stereo methods estimate disparity by considering all pixels simultaneously, instead of treating each pixel or patch independently. The goal is to find a disparity map that not only matches the image evidence but is also consistent and smooth across the entire image.

They achieve this by defining an energy function with two main components:

• Data term: Measures how well a proposed disparity agrees with the appearance of the two images. This typically comes from the same matching costs used in local methods (SSD, SAD, NCC, etc.).
• Smoothness term: Encourages neighboring pixels to have similar disparities so the surface is spatially coherent. This term allows the algorithm to fill in textureless regions and reduce noise. The smoothness penalty is reduced at image edges so depth boundaries can be preserved.

The algorithm then finds the disparity map that minimizes the total energy, balancing image evidence with spatial coherence.

Example of an energy function (conceptual form)

E(d) = Σ_p D(p, d_p) + λ Σ_(p,q∈neighbors) S(d_p, d_q)

Where:
• D(p, d_p) is the data cost at pixel p for disparity d_p.
• S(d_p, d_q) is the smoothness penalty between neighboring pixels p and q (e.g., 0 if equal, small penalty if similar, large penalty if very different).
• λ controls the trade-off between matching accuracy and smoothness.
• The summations run over all pixels and all neighbor pairs.

This expresses the global idea: the best disparity map is the one with the lowest total energy, not just the best per-pixel score.

How the optimization is solved

Because the energy depends on the whole image, global methods use more advanced optimization techniques, such as:

• Dynamic Programming (efficient but often 1D, used along each scanline)
• Graph Cuts (very strong global optimization, good at preserving edges)
• Belief Propagation (message passing to reach a consistent global solution)

These methods produce smoother, more accurate disparity maps than local area-based matching, especially in:

• low-texture regions
• repetitive patterns
• noisy or low-light conditions

The trade-off is significantly higher computational cost, making them slower but much more reliable for applications requiring accuracy over speed.

# Lecture 4 – Camera calibration & Epipolar geometry

## What “projection” means in computer vision

## Projection Matrix

Projection = mapping a 3D point in the world onto a 2D point in the camera image.

A real camera takes a point
(X, Y, Z) in 3D space
and produces image coordinates
(u, v) on the sensor.

This mathematical mapping is called the camera projection.

What the projection matrix is (monocular camera)

For a single camera, the projection matrix is:

P = K [ R | t ]

Where:

K = intrinsic matrix

Contains camera-specific internal parameters:

- focal length fx, fy

- principal point cx, cy

- skew (usually 0)

[R | t] = extrinsic parameters

Describe the camera’s pose in the world:

- R = rotation from world to camera

- t = translation from world to camera

What P does

It converts a homogeneous 3D point:

[X, Y, Z, 1]ᵀ

into a 2D pixel coordinate:

[u, v, 1]ᵀ

via

x = P X

This is the complete camera model used in projection and calibration.

How this differs from the fundamental matrix

Projection matrix P

- Maps 3D → 2D

- Used for single-camera geometry

- Contains intrinsics + extrinsics

- Results from camera calibration

- Central to pinhole camera model

Fundamental matrix F

- Maps a point in one image to a line in another image

- Describes epipolar geometry between two cameras

- Does not map 3D → 2D

- Does not contain intrinsics

- Works with uncalibrated stereo pairs

So:

Projection matrix = single-camera 3D-to-2D model
Fundamental matrix = two-camera relational constraint

They are completely different objects.

Homography vs Projection Matrix vs Fundamental Matrix (super short)

| Matrix | Maps | Used for | Contains |
| --- | --- | --- | --- |
| Projection matrix P | 3D → 2D | Camera calibration, reconstruction | Intrinsics + extrinsics |
| Homography H | 2D → 2D (planar scenes) | Image warping, stitching | No 3D info |
| Fundamental matrix F | image1 point → epipolar line in image2 | Uncalibrated stereo | Epipolar geometry (no intrinsics) |

Final answer

When they say projection, they mean the pinhole camera projection, implemented by the projection matrix P = K[R|t], which converts 3D world points into 2D image points.
It has nothing to do with the fundamental matrix.

If you want, I can write a 2–3 sentence exam-style explanation.

### What a homography actually is

A homography is a 3×3 matrix that maps points in one image to their exact locations in another when all points lie on a single plane or when the camera undergoes pure rotation.
In these cases, the two images differ only by a 2D projective warp.

Typical uses: image stitching, rectifying a wall or document, projecting textures onto a flat surface, AR on a table, etc.

The equation is x' = Hx (in homogeneous coordinates).

A homography has 8 degrees of freedom because it has 9 parameters, but scale does not matter.

Why only 4 points?
Because each point correspondence provides two independent linear equations.
To determine 8 unknowns, you need 4 correspondences (4×2 = 8 equations).
So the minimal solver uses 4 points.

### What the fundamental matrix is

The fundamental matrix relates point correspondences between two images when the cameras translate, creating parallax.
It does not warp one image into another. Instead, it enforces the epipolar constraint:

x'ᵀ F x = 0
Every correspondence must satisfy this bilinear constraint.

F encodes the epipolar geometry: the epipoles, the epipolar lines, and the directions of motion. It works for any 3D scene, not just planes.

### Why F also appears to have 8 degrees of freedom

F also has 9 parameters, but again scale does not matter.
So superficially you might think “8 DOF, so 4 points should be enough.”

But there is an additional constraint:

det(F) = 0
F must be rank-2.

This is a non-linear constraint, and it reduces the degrees of freedom of valid fundamental matrices to 7.

Because of this nonlinear constraint, each correspondence gives only one independent equation (x'ᵀ F x = 0 is a single scalar equation).

To solve for 7 unknowns, you could use 7 points (the 7-point algorithm).
However, the 7-point algorithm is numerically unstable and can produce up to 3 solutions.

Therefore, in practice we use the 8-point algorithm:
8 correspondences → 8 linear equations → initial F → then project it to rank-2 (enforce det(F) = 0 by SVD).

So the real reason F needs 8 points is not its number of DOF,
but the fact that each correspondence provides only one usable equation.

### Summary of the real difference

Homography:
• Assumes planar scene or pure rotation
• Maps image → image
• 2 equations per point → 4 points needed
• Used for stitching, rectification, flat surfaces

Fundamental matrix:
• Handles general 3D scenes with translation
• Encodes epipolar geometry, not an image warp
• 1 equation per point → 8 points needed (or 7 for a special nonlinear solver)
• Used for stereo vision, 3D reconstruction, camera pose

The Essential Matrix is a 3×3 matrix that encodes the epipolar geometry between two calibrated cameras and relates corresponding normalized image coordinates.

The Fundamental Matrix is a 3×3 matrix that encodes the epipolar geometry between two uncalibrated cameras and relates pixel coordinates directly.

They are different because the Essential Matrix uses known camera intrinsics and works in normalized camera coordinates, while the Fundamental Matrix works in raw pixel space and does not assume any knowledge of the cameras’ internal calibration.

Levenberg–Marquardt – used for distortion:

From lecture:

How to calibrate for the distortion coefficients: First do DLT for the Projection Matrix. Then form a non linear least square problem that includes both the linear projection and the non-linear distortion using the Levenberg Marquardt algorithm

Levenberg–Marquardt
A nonlinear least-squares optimizer that blends gradient descent and Gauss–Newton to stably and efficiently minimize reprojection error in camera calibration.

## Essential and Fundemental Matrix

E is “singular” because its determinant is zero and its rank is 2, a structural property resulting from its construction as a skew-symmetric translation matrix multiplied by a rotation.

The essential matrix must be singular so that it encodes a physically valid epipolar geometry, ensures the epipolar constraint holds, and allows recovery of the camera motion (R and t).

The essential matrix has 5 degrees of freedom because it is defined by a rotation (3 DoF) and a translation direction (2 DoF), while the fundamental matrix has 7 degrees of freedom because it represents the same geometry in uncalibrated pixel coordinates and is defined up to scale minus one rank-2 constraint.

The fundamental matrix has 7 degrees of freedom because it is a 3×3 matrix with 9 parameters, minus 1 for scale (homogeneous representation) and minus 1 for the rank-2 constraint, leaving 7 independent parameters.

HOMOGENOUS LINE

A homogeneous line is a 3-vector ((a, b, c)) that represents the 2D line whose points ((x, y, 1)) satisfy the equation (ax + by + c = 0).

# Fundemental matrix algorithm
8-Point Algorithm (Fundamental Matrix Estimation)

### 1. Normalize the image points

Transform the pixel coordinates xand x'into a normalized coordinate system using similarity transforms Tand T'.
This improves numerical stability.

x=Tx,x'=T'x'

### 2. Solve for Fusing 8 or more correspondences

- Build the linear system x'T F x=0for all point pairs.

- Solve using SVD: the last column of Vgives F.

- Enforce rank 2 (det = 0) by zeroing the smallest singular value via a second SVD.

### 3. De-normalize to get the actual fundamental matrix

F=T'T F T

This transforms the normalized solution back to the original pixel coordinate system.

# One-sentence summary

Normalize points → solve a linear system via SVD → enforce rank-2 → de-normalize to obtain the final fundamental matrix.

## Lens Distortion

Real lenses deviate from the ideal pinhole model. Light bends more at the edges, causing straight lines to appear curved. The most common types are barrel distortion (lines bulge outward), pincushion distortion (lines curve inward), and fisheye distortion (extreme wide-angle warping). Distortion grows with distance from the image center, so it is modeled as a radial effect.

Distortion is usually described with a radial polynomial applied to the normalized camera coordinates. The idea is that the farther a point is from the center, the more the polynomial scales it. This allows the model to capture how much the lens pushes or pulls points radially.

Calibration estimates these distortion coefficients together with the camera intrinsics. The typical process is: first compute an initial projection matrix using DLT, ignoring distortion. Then refine the parameters using a non-linear least-squares optimization (often Levenberg–Marquardt) that minimizes reprojection error while including both the linear projection model and the non-linear distortion terms. This gives accurate values for focal length, principal point, and the distortion coefficients.

Once the distortion model is known, images can be undistorted by mapping each pixel back to where it would appear in an ideal pinhole camera. This produces images where straight lines are straight and geometric measurements become reliable. Distortion correction is essential for stereo vision, 3D reconstruction, feature matching, and any task requiring accurate geometry.

## Workflow to handle lens distortion

Estimate intrinsics + distortion:

- Capture several images of a calibration pattern (e.g. checkerboard) from different poses.

- For each image, detect all calibration points in image coordinates.

- For each image, know the corresponding 3D pattern coordinates.

- Use DLT (or similar) to get an initial camera matrix without distortion.

- Set initial distortion coefficients k1, k2, … = 0.

- Define an error function = sum over all images of (observed image points − projected 3D points after applying camera matrix + distortion model).

- Run non-linear least squares (e.g. Levenberg–Marquardt) to minimize this error with respect to:

  - intrinsics (focal lengths, principal point),

  - extrinsics for each image (pose),

  - distortion coefficients k1, k2, …

- Store final intrinsics and distortion parameters.

Undistort a new image:

- For each pixel in the undistorted image grid, compute its ideal normalized coordinates.

- Apply the distortion model with the calibrated k1, k2, … to find where this point would land in the distorted image.

- Sample (interpolate) the color at that distorted location in the original image.

- Write it to the undistorted image.

# Lecture 5

# Why do we need 3D Point Clouds?

- The real world is 3D, so a 2D image loses depth, distance, and true geometry.

- Many tasks (navigation, manipulation, obstacle avoidance) require knowing where things are in space, not just their appearance in 2D.

- 3D point clouds give explicit shape, size, and spatial structure of objects and scenes.

- They enable reasoning about volumes, surfaces, and distances, which 2D pixels cannot express.

### However:

- Working directly in 3D is computationally expensive (more data + more complex geometry).

- Traditional image-based features (SIFT, SURF, ORB) do not transfer well to unstructured 3D point clouds because they assume pixel grids and well-defined neighborhoods.

### Bonus:

- Point-cloud processing requires specialized methods (e.g., ICP, RANSAC planes, 3D descriptors, deep models like PointNet).

# What is “Pose”?

Pose describes the full rigid-body transformation — rotation and translation — that aligns one coordinate system or point cloud with another.
It tells you where an object is and how it is oriented in 3D space.

- In point-cloud terms: pose is the transformation that maps one 3D model to another view of the same object or scene.

- In vision terms: pose is the camera’s position and orientation relative to the world or a known 3D structure.

In short: pose = where + which way an object (or camera) is in 3D.

# Why is 3D Pose Estimation Important?

3D pose estimation is crucial because autonomous systems must understand both where objects are and how they are oriented in space to interact with them safely and intelligently.

### Key applications include:

- Robot Grasping & Manipulation:
Robots must know the exact 3D position and orientation of objects to grasp, rotate, or assemble them reliably.

- Quality Inspection:
Precise pose allows comparing scanned parts to CAD models, detecting misalignments, deformations, or defects.

- Augmented Reality:
AR systems need accurate camera pose to correctly anchor virtual objects in the real world.

- Progressive Map Building (SLAM):
Robots estimate their own pose while constructing a consistent 3D map of the environment.

In short: pose estimation enables meaningful physical interaction and spatial understanding in real-world systems.

## KD-trees

KD-trees reduce nearest-neighbor search from O(N) to about O(log N), turning alignment steps from quadratic to near-linear, which is a massive speedup for large point clouds.

A KD-tree works by recursively splitting points along alternating axes (x, y, z), forming a balanced binary tree. During a nearest-neighbor search, large regions of space are quickly pruned, so only a small subset of points needs to be checked, making the search much faster.

KD-trees are used in SIFT after the descriptors are computed, to speed up feature matching.
Each SIFT descriptor is a 128-D vector, and matching requires finding the nearest neighbor descriptor in another image.
Instead of comparing every descriptor to every other one (slow), a KD-tree (or multiple KD-trees like in FLANN) organizes descriptors so nearest-neighbor lookup becomes much faster.

In one sentence:
SIFT uses KD-trees to quickly find nearest-neighbor matches between 128-D descriptors, massively accelerating feature matching.

in high-dimensional KD-trees (like for SIFT’s 128-D descriptors), the split axis is chosen as the dimension with the highest variance.

For SIFT descriptors:

- Each descriptor is a 128-dimensional vector.

- A KD-tree node chooses the dimension with the largest spread (variance) across all descriptors at that node.

- It splits the data at the median value along that dimension.

Why do this?

Because:

- Some descriptor dimensions have almost no variation.

- Splitting along a low-variance axis creates a poor, unbalanced tree.

- Splitting along the highest-variance dimension gives the most balanced, most effective partitions.

One-sentence exam answer

KD-trees for SIFT choose the split axis as the descriptor dimension with the highest variance, then split at the median, to produce balanced partitions that speed up nearest-neighbor matching.

How KD-tree backtracking works (short)

When searching for the nearest neighbor, the KD-tree first follows the branch that contains the query point (the “good” side).
It keeps a record of the current best distance found so far.

After reaching a leaf, the algorithm backtracks and decides whether it must explore the other side of a split:

The check:

If the distance from the query point to the splitting plane is smaller than the current best distance,
→ the nearest neighbor could be on the other side.
So the algorithm visits the other branch.

If the distance to the plane is larger than the best distance,
→ the other side cannot contain a closer point and is skipped.

This is sometimes called the radius test:

- “radius” = current nearest-neighbor distance

- If the split plane is within this radius, search the other side.

## Singular value decomposition

How to go from x’Fx = 0 to Af = 0 to build the fundamental matrix FThe equation "x'ᵀ F x = 0" is the epipolar constraint, but it can also be rewritten directly as a linear algebra equation in the nine unknown entries of F. To see this, write the points in homogeneous coordinates as x = (x, y, 1) and x' = (x', y', 1). Also write the fundamental matrix F using its nine entries F11, F12, F13, F21, F22, F23, F31, F32, F33. When you multiply F with x, you get three expressions that are linear combinations of the F-entries. Then, when you multiply x'ᵀ with that result, you get a single scalar equation. If you expand it fully, you obtain something like:
x * x' * F11 + y * x' * F12 + 1 * x' * F13 + x * y' * F21 + y * y' * F22 + 1 * y' * F23 + x * F31 + y * F32 + 1 * F33 = 0.

This is just a linear equation where the coefficients depend on the image coordinates (x, y, x', y'), and the unknowns are the nine entries of F. If we collect the nine F-entries into a vector
f = [F11, F12, F13, F21, F22, F23, F31, F32, F33]ᵀ,
and we collect all the coordinate-based coefficients into a row vector based on one correspondence, then the scalar equation above becomes exactly a dot product between that row and the vector f.

When we have multiple correspondences, we simply stack all such rows into a matrix A, so that all epipolar constraints together can be written as the matrix equation:
A * f = 0.

This is why we say it is a “direct algebraic rewriting”: the original geometric constraint x'ᵀ F x = 0 is expanded into plain algebra, reorganized, and expressed in matrix–vector form, but it is still the exact same equation.

Kabsch algorithm / Procrustes Analysis

We move both centroids to 0 so that the alignment only estimates rotation, not rotation mixed with translation. Subtracting the centroids removes the global offset between the point clouds, ensuring that the algorithm compares the actual shapes and directions of the points rather than their absolute positions; after finding the optimal rotation in this centered space, the translation can be added back cleanly at the end.

REFLECTION
Yes — that “S matrix in the middle” is exactly the reflection-fixing step from the Kabsch algorithm, and it’s completely standard.

When we compute the optimal rotation using SVD of the covariance matrix,
C = U Σ Vᵀ,
the rotation candidate is R = U Vᵀ.

However, this matrix is not guaranteed to be a proper rotation (determinant +1). Sometimes det(U Vᵀ) = –1, which means the transformation contains a reflection (a mirror flip). A rigid-body transformation is not allowed to reflect — it must be a pure rotation.

To fix this, we insert a matrix S between U and Vᵀ:

S = diag(1, 1, det(U Vᵀ)).

This matrix is:
– diag(1, 1, 1) if no reflection is present,
– diag(1, 1, –1) if a reflection is detected.

The corrected rotation is:

R = U S Vᵀ.

By flipping only one axis (the one associated with the smallest singular value), S removes the reflection while disturbing the alignment as little as possible. This gives us a true rotation with determinant +1.

One-sentence summary:
We insert the matrix S = diag(1,1,det(UVᵀ)) between U and Vᵀ to remove reflections from the SVD result and ensure the final transformation is a valid rotation, not a mirror flip.

QUESTION:

• Why ICP needs to employ Kabsch algorithm at every iteration?

ICP must run the Kabsch algorithm at every iteration because each alignment step changes the nearest-neighbor correspondences, and every new set of correspondences defines a new optimal rotation and translation that must be recomputed to keep improving the alignment.

## Global alignment

## Spin Images — How They Work

Spin Images describe the shape of the local surface around a point by using its surface normal as a reference direction.

How it works:

- Take a point p and compute its normal (by fitting a small plane to its neighbors).

- For every neighbor point q, compute:
• α = perpendicular (sideways) distance from q to the normal axis
• β = distance along the normal (forward/backward)

- Insert (α, β) into a 2D histogram grid.

This 2D histogram (like an image) captures how points lie around p in a rotation-invariant way — because spinning around the normal doesn’t change α or β.

Why it works:
The normal provides a stable orientation, and the histogram encodes the “shape profile” around the point.

Descriptor size: Depends on grid resolution (e.g., 16×16 bins).

# PFH — Point Feature Histogram (Full Descriptor)

PFH describes how local surface patches relate to each other by comparing every pair of points in a neighborhood.

Why 3 features?
Because the relative geometry between two oriented surface patches in 3D has three degrees of freedom.
PFH computes exactly three angular values (α, φ, θ) describing:

- How the normals tilt relative to each other

- How each normal relates to the line joining the points

- The “twist” of one normal around the connection line

How it works:

- For a point p, find k neighbors.

- For every pair of neighbors (i, j), compute the 3 angular features (α, φ, θ).

- Each of the 3 angles is quantized into 5 bins.

- This gives a 5 × 5 × 5 = 125-bin histogram.

- Add all angle triples into the histogram.

What the descriptor captures:
The full local curvature, twist, and orientation relationships — extremely descriptive.

Why it is slow:
PFH uses all point–point pairs → O(k²) computations.

FPFH — Fast PFH (Approximation of PFH)

FPFH achieves almost the same descriptive power as PFH but much faster.

How FPFH works:

- For the center point p, compute angular features only between p and each neighbor.
These are stored in a 33-bin histogram (the Simplified Point Feature Histogram, SPFH).

- Gather the SPFHs of the neighbors and blend them (weighted by distance).

- The result is the FPFH descriptor, also 33 values.

Why it is MUCH faster:

• PFH compares all pairs of neighbors: k(k−1)/2 operations (quadratic). Meaning it must examine every possible neighbor-to-neighbor relationship around the point, which becomes extremely expensive as the number of neighbors grows.

• FPFH compares only center–neighbor pairs: k operations (linear).
• The missing neighbor–neighbor relationships are approximated by reusing neighbors’ SPFHs.

Why 33 bins instead of 125:
• PFH explicitly models all pairwise angular relationships, and therefore builds a joint histogram over three angular features (α, ϕ, θ).
Because it treats these features jointly, it needs a 5×5×5 grid → 125 bins.
• FPFH does not build a joint 3D histogram. Instead, it stores each angular feature separately in its own 11-bin histogram.
3 angular features × 11 bins = 33.
• In other words, FPFH uses marginal histograms for each feature instead of a full 3D histogram over all features.
This dramatically reduces size while retaining most of the discriminative power.

Intuition for why PFH needs 125 bins:
PFH wants to understand how the three angular features behave together.
It is not enough for PFH to know “α is usually small” or “θ varies a lot.”
PFH wants to know things like:

• “When α is small, is ϕ also small?”
• “When θ is large, what range does ϕ usually take?”
• “Do certain combinations of angles happen often?”

This is a joint distribution over three variables.
To store all combinations, PFH builds a 3D histogram:

- 5 bins for α

- 5 bins for ϕ

- 5 bins for θ

When you combine these you get 5 × 5 × 5 = 125 possible angle combinations.
Each combination gets its own bin.
That’s why PFH is so big: it tracks every possible relationship between the angles.

Intuition for why FPFH only needs 33 bins:
FPFH says:
“Instead of storing all angle combinations, I’ll store the distribution of each angle separately.”

Meaning:
• one histogram for α
• one histogram for ϕ
• one histogram for θ

Each with 11 bins → 33 numbers total.

Instead of asking “how do α, ϕ, and θ occur together?”,
FPFH asks three simpler questions:
• “What values does α take?”
• “What values does ϕ take?”
• “What values does θ take?”

This removes the combinatorial explosion.
It throws away some detailed co-occurrence information,
but keeps most of the geometric identity of the surface—
good enough for fast registration.

Short intuitive summary:
PFH stores a big 3D histogram that captures how all three angular features interact → 125 bins.
FPFH stores three small 1D histograms, one for each feature → 33 bins.
PFH captures detailed angle relationships; FPFH captures general angle behavior.
FPFH is much smaller and faster, and usually still discriminative enough.

Descriptor size:
• 33 bins (FPFH) vs. 125 bins (PFH)

Trade-off:
• FPFH is slightly less detailed (no full 3D joint distribution), but fast enough for real-time global registration.

# ⭐ Why these descriptors need normals

Normals give each point a local orientation frame.
Without normals, you can’t describe curvature or how surfaces bend.
These descriptors rely on normals to understand:

- shape

- slope

- twist

- curvature

- local surface variation

Normals are computed by fitting a small plane to each point’s neighborhood (using PCA). Only surface point clouds (like LIDAR scans) make sense for these descriptors

| Descriptor | Uses normal? | Uses all pairs? | Bins | Speed | Notes |
| --- | --- | --- | --- | --- | --- |
| Spin Image | Yes | No | 2D grid (e.g., 16×16) | Medium | Uses distances relative to normal |
| PFH | Yes | Yes (full pairs) | 125 | Slow (O(k²)) | Most detailed |
| FPFH | Yes | No (approx pairs) | 33 | Fast (O(k)) | Approx of PFH |

# Lecture 6

## Why is ML important for Perception of

Autonomous Systems?

• Create initial object hypotheses

– input to pose estimation?

• Abstract representation of scene

- Improve robustness: ML handles noise, lighting changes, and occlusions better than hand-crafted methods.

- Learn useful features: ML extracts the right features automatically instead of relying on manual design.

- Enable high-level understanding: ML allows recognition, segmentation, and prediction needed for planning and navigation.

## K-means

How it works

- Initialize k cluster centers (usually randomly).

- Assignment step: Assign each data point to the nearest center (Euclidean distance).

- Update step: Recompute each center as the mean of its assigned points.

- Repeat steps 2–3 until convergence (centers stop moving or max iterations reached).

K-means minimizes the sum of squared distances from points to their assigned cluster center.

Pros

- Simple and fast, works well for large datasets.

- Easy to implement and interpret.

- Often gives good results when clusters are well separated and roughly spherical.

- Scales well to high dimensional data.

Cons / Limitations

- Requires k beforehand (number of clusters must be chosen).

- Sensitive to initialization → can converge to bad (local) minima.

- Prefers spherical, equally sized clusters → struggles with irregular shapes or different densities.

- Not robust to outliers (means shift easily).

- Uses Euclidean distance, so scaling of features matters.

Question

When performing k-means clustering on a 3D point cloud, in which feature space should the clustering operate, and how many dimensions should be used?

Answer

K-means does not have to operate only on XYZ coordinates.
You can cluster points in any feature space that helps separate meaningful structures in the scene.

Possible dimensionalities:

- 3D (X, Y, Z): Clusters purely by geometry; good for separating spatial regions but fails when objects overlap or touch.

- 6D (X, Y, Z, R, G, B): Uses both position and color; helpful when objects differ visually.

- 6D (X, Y, Z, L, U, V) or (X, Y, Z, H, S, V): Color in perceptually uniform spaces improves clustering stability.

- >6D: Add texture, gradients, intensity, normals, or other features for richer segmentation.

In short: the dimensionality depends on which features best distinguish the objects or surfaces you want to segment. More informative features → better clusters.

## Mean Shift — Summary

How it works

Mean Shift is a mode-seeking clustering algorithm that identifies clusters as peaks (modes) in the estimated density of data points.

- Choose a kernel and bandwidth (h).

- For each point:

  - Place a window (radius = bandwidth) around the point.

  - Compute the mean of all points inside the window.

  - Shift the window center to this mean.

  - Repeat until the shift becomes very small → convergence to a mode.

- Cluster assignment:
Points that converge to the same mode are grouped into the same cluster.

The algorithm essentially performs gradient ascent on a kernel density estimate, moving points toward regions of highest density.

Intuition

Mean Shift works like sliding a window uphill on a density landscape:
every iteration moves the point toward a local maximum (a mode).
Points that climb to the same peak belong to the same cluster.

Pros

- No need to choose the number of clusters (k) — the number of modes emerges automatically.

- Works well with arbitrary, non-convex cluster shapes.

- Robust to outliers, since far-away points have little influence.

- Simple to implement and conceptually intuitive.

Limitations / Cons

- Computationally expensive: each point performs iterative updates and neighbor searches → slow for large datasets.

- Choosing bandwidth h is critical:

  - Too small → many tiny clusters (over-segmentation)

  - Too large → merge distinct clusters (under-segmentation)

- High-dimensional feature spaces make KDE and neighbor searches inefficient (“curse of dimensionality”).

- Convergence may be slow in flat or noisy density regions.

When to use

- When cluster shapes are irregular and not spherical (unlike k-means).

- When you want clustering without specifying k.

- When dataset size is moderate (not millions of points).

✔ One-sentence summary

Mean Shift clusters by iteratively shifting each point toward the nearest density peak using kernel density estimation; it finds arbitrary-shaped clusters without choosing k, but is slow and sensitive to bandwidth.

2D Image Segmentation by Mean Shift — Explanation

Mean Shift can segment an image by clustering pixels in a combined feature space that includes both color and spatial position. Each pixel becomes a point in a higher-dimensional space, and Mean Shift finds groups of pixels that converge to the same density peak (mode), forming a segment.

1. Compute features for each pixel
Each pixel is represented as a feature vector, for example:

- x,y,L,u,v(position + color in Luv*)

- L,a,b,texture

- or any combination of color, gradients, texture, etc.

2. Set kernel size for features Kfand position Ks	
Two bandwidths are used:

- Ks: spatial radius (how far in the image we look)

- Kf: feature radius (how similar colors/features must be to influence the shift)

These control how smooth or fine the segmentation becomes.

3. Initialize windows at each pixel
Each pixel starts its own Mean Shift procedure, meaning every pixel begins moving toward a nearby high-density region in feature space.

4. Perform Mean Shift until convergence
For each pixel:

- Look at all points within the spatial + feature bandwidths

- Compute the mean of those points

- Shift the window to that mean

- Repeat until the shift is very small (mode reached)

5. Merge windows that converge to nearby modes
After convergence, pixels whose trajectories end near the same mode are assigned to the same cluster/segment. Neighboring modes within bandwidth thresholds Ksand Kfare merged.

Result:
Large, coherent regions in the image (with similar color and local structure) are detected automatically, without specifying the number of clusters.

3D Point Cloud Segmentation by Mean Shift — Explanation

When applying Mean Shift to a point cloud, the first question is:

In what feature space should the clustering take place?

A point cloud is naturally in 3D geometric space (X, Y, Z),
but you can add any additional features that help separate meaningful structures.

The dimensionality defines how each point is represented during clustering.

Dimensionality options

1. Dimensionality = 3 → (X, Y, Z)

Clustering only on geometry.

- Separates objects that are spatially separated

- Fails when objects are close or overlap

- No color or appearance information used

2. Dimensionality = 6 → (X, Y, Z, R, G, B)

Combine geometry + RGB color.

- Good when different objects have different colors

- More robust to overlapping shapes

- Sensitive to lighting

3. Dimensionality = 6 → (X, Y, Z, L, U, V)

Use LUV or similar perceptual color spaces.

- Better color separation than RGB

- More perceptually uniform

- Useful for natural scenes and outdoor point clouds

4. Dimensionality = 6 → (X, Y, Z, H, S, V)

Use HSV color features.

- Separates objects by hue even when brightness varies

- Useful in scenes with variable illumination

5. Dimensionality > 6 → (X, Y, Z, H, S, V, gradients, texture, normals, …)

Add additional descriptors such as:

- Surface normals

- Curvature

- Texture

- Intensity

- Gradient magnitude

- Time history or motion cues

More features help separate objects with similar color and geometry,
but increase computational cost and the curse of dimensionality.

Key point

Mean Shift works in any feature space — not just the raw 3D coordinates — and choosing the right dimensionality determines how well it can segment real structures in the point cloud.

# Lecture 7 – Histogram/ Kalman filter

## Posterior vs Prior distribution

- Prior = your belief before seeing the new measurement (prediction from the motion model).

- Posterior = your belief after incorporating the measurement (prior updated using Bayes’ rule).

If you want the ultra-compact intuition:
Prior = “what I expect before looking.”
Posterior = “what I believe after looking.”

# Lecture 8 – Classification

## Eigenfaces:

1. Training stage: Create Eigenfaces

Using a dataset of aligned face images:

- Convert each image to a vector.

- Compute the mean image.

- Subtract the mean from every face (center them).

- Run PCA on the centered faces.

- Keep the top keigenvectors → these are the eigenfaces.

- For each training person, compute their coefficient vector:

ci=vi⊤(x-mean)

That coefficient vector is the person’s position in “face space.”

2. Registration (enrolling a person)

When a new person is added to the system:

- Take their face image.

- Subtract the mean image.

- Project onto eigenfaces → compute coefficients c1,c2,…,ck.

- Store this coefficient vector in the database under that person’s name.

This is the person’s signature or embedding.

3. Recognition (identifying a new face later)

When you get a new image of someone:

- Subtract the mean image.

- Project onto the same eigenfaces → get a new coefficient vector.

- Compare this vector to all registered vectors using Euclidean distance.

Closest match → same identity.

If the distance is too large → “unknown person.”

One-sentence summary (your style):

Train: build eigenfaces; Register: store each person’s coefficient vector; Recognize: project the new face and find the closest stored vector.

k-NN Training vs Inference Summary (for notes)

- Training cost:
k-NN has no real training. It only stores all (N) data points → O(N).

- Inference (testing) cost:
For each new sample, k-NN must compute the distance to every stored point.
Each distance is over (d) features → O(Nd).

- Why not O(k)?
You must compare against all (N) points before selecting the (k) nearest, so inference cannot be just O(k).

- Conclusion:
Inference is much slower than training.
The statement “k-NN requires similar time for training and testing” is false.

## Distance metrics (k-NN)

Shortest possible explanations:

- Euclidean: straight-line distance between two points.

- Manhattan: distance measured by moving horizontally + vertically (like city blocks).

- Mahalanobis: distance that accounts for variance—penalizes differences more along stable directions and less along noisy ones.

Euclidean distance:
d = sqrt( Σ (xi − yi)² )

Manhattan distance:
d = Σ |xi − yi|

Mahalanobis distance:
d = sqrt( (x − y)ᵀ Σ⁻¹ (x − y) )

# Lecture 9 – Visual Odometry

## Optical Flow

### Lucas–Kanade (Sparse Optical Flow)

Lucas–Kanade computes flow only at selected feature points, not for every pixel.
It assumes that nearby pixels share the same motion, and it solves a small least-squares problem inside a window around each feature.

Characteristics:
• Computes motion at a sparse set of good features (corners).
• Assumes small motion between frames.
• Very fast, works well for feature tracking (e.g., KLT tracker).
• Fails in flat regions because there is no texture to track.
• Often used in SLAM, visual odometry, point tracking in robotics.

### Dense Optical Flow (Gunnar–Farnebäck)

Farnebäck’s method computes flow for every pixel, not just features.
Instead of assuming constant intensity inside a small window, it models each neighborhood as a quadratic polynomial. By seeing how this polynomial changes from one frame to the next, it estimates the local motion field.

Characteristics:
• Produces a dense motion map (vector per pixel).
• More robust to smooth intensity changes.
• Handles larger deformations better than Lucas–Kanade.
• Slower than sparse methods, but still fast enough for real-time use in many applications.
• Often used for segmentation, motion analysis, background subtraction, and video effects.

### What pyramid versions actually mean

Both Lucas–Kanade and Farnebäck assume small motion between frames.
If the motion is large, they will fail because a point moves too far to be matched.

A pyramid solves this by creating multiple downscaled versions of the image:

- Start with a tiny version of the image → motion appears small.

- Estimate flow there.

- Upsample flow to the next (larger) resolution and refine it.

- Continue until you reach the original image.

This lets the algorithm track large motions by handling them first at low resolution where they appear small, then refining at higher resolutions.

This is why pyramids are essential for optical flow in real videos.

### When to use which

Lucas–Kanade (sparse):
• Use when you only need motion of specific points (tracking, odometry).
• Very cheap and stable, but not dense.

Farnebäck (dense):
• Use when you need motion everywhere (segmentation, flow fields, video analysis).
• More computationally heavy but gives far richer information.

### VO vs. SFM vs. VSLAM — Summary

SFM (Structure from Motion)

- Solves for camera poses and 3D point positions.

- Uses the projection equation:

where π is the projection function.

- Goal: recover full 3D structure.

VO (Visual Odometry)

- Only estimates camera pose between frames.

- Does not solve for complete 3D structure.

VSLAM (Visual SLAM)

- Like VO but adds state estimation + loop closures.

- Re-observations create constraints that optimize the global trajectory.

## Used in Structure for motion (SFM):

Bundle adjustment = a nonlinear optimization that jointly refines camera poses and 3D point positions to minimize reprojection error.

In other words:
It adjusts all poses + all 3D points so that the projected points match the image measurements as closely as possible.

Shortest form:
“Global refinement of poses and 3D structure by minimizing reprojection error.”

## Orientation

Rotation Matrix:
A 3×3 orthonormal matrix representing orientation. It is unique and avoids gimbal lock, making it robust for long robot trajectories. However, rotation matrices are unintuitive for humans and difficult to interpolate or perturb directly, because small numerical changes can break orthonormality.

Euler Angles:
A minimal and intuitive parameterization using yaw, pitch, and roll. Easy for human interpretation, but suffers from gimbal lock and depends on rotation order, making it unsuitable for general 3D control or estimation tasks.

Axis–Angle:
Represents a rotation by an axis direction and a rotation magnitude. It is minimal, free from gimbal lock, and well-behaved for small perturbations. However, rotation magnitude does not scale linearly, and conversions require exponential/logarithmic maps.

Quaternions:
A 4-parameter unit vector representing 3D rotation. They avoid gimbal lock, support smooth interpolation (SLERP), and are computationally efficient. Their downside is that the representation is not intuitive and requires normalization and mathematical care.

Gimbal lock:
Gimbal lock happens when two of the three rotation axes in an Euler-angle system become aligned.
When this occurs, you lose one degree of rotational freedom—the system can no longer rotate independently around all three axes.

In other words: Two rotation axes collapse into one → one rotation becomes impossible.
This makes certain orientations unrecoverable or unstable for control/estimation.

It is a fundamental flaw of Euler angles, not of 3D rotation itself.

# Relative Pose Estimation – 3D Point Cloud Correspondences (Clean Explanation)

This slide describes how to estimate the camera motion when both frames provide 3D points. If we know which 3D points in frame k match the 3D points in frame k–1, we can solve directly for the rotation R and translation t between the two frames. This is called 3D registration.

Typical methods for solving this include PCA, SVD, RANSAC, ICP, or combinations of them.
SVD is the standard closed-form solution when correspondences are known, while RANSAC helps reject outliers.

When correspondences are available, the goal is to find the rigid transformation (R, t) that best aligns the two point clouds. This is done by minimizing the sum of squared distances between each point in frame k–1 and the corresponding point in frame k after applying R and t.

RANSAC only needs three point pairs to hypothesize a rigid transformation, because three non-collinear points are enough to uniquely determine a 3D pose. Each RANSAC iteration picks three random pairs, computes R and t, and checks how many points fit that motion.

Relative Pose estimation - 2D Image Points

# Scale Problem – Questions and Answers

1. Why is the essential matrix calculated only up to scale?
Because 2D image correspondences do not contain any absolute depth information. The epipolar geometry equations depend only on the direction of translation, not its true magnitude. Therefore, the translation vector extracted from the essential matrix is determined only up to an unknown multiplicative factor.

2. What does it mean that the translation t in [R | t] is also up to scale?
It means that the direction of motion between the two frames is correct, but the actual distance travelled is unknown. A small motion close to the scene and a large motion far away can produce identical 2D projections, so the real scale cannot be recovered from monocular images alone.

3. How can we recover the true scale?
To recover scale, we need at least one additional source of metric information. Common approaches include:

- Using stereo cameras (baseline gives metric depth)

- Using an IMU (acceleration integration gives motion magnitude)

- Using known object sizes in the scene

- Tracking features over multiple frames and triangulating 3D structure

- Using wheel odometry, LiDAR, or GPS if available

4. What does “a relatively accurate movement between frames fk and fk−1” mean?
It means that without extra information, we can only recover the shape of the motion (rotation and direction of translation). To obtain the real distance moved between frames, we must estimate or measure the absolute scale using one of the methods above.

# Answers to “Some Notes” Slide

1. Why are 2D–2D and 3D–2D better than 3D–3D?

- 3D–3D requires accurate depth for both frames, which is often noisy or unreliable.

- Depth errors quickly produce large pose errors because the algorithm depends on precise 3D point alignment. Subtracting two noisy depth measurements amplifies error because each depth may be wrong in a different direction. When you subtract them, the noise does not cancel — it adds. In 3D–3D this doubled noise affects the pose estimate, while 3D–2D uses only one noisy depth value and therefore has half the uncertainty.

- In contrast, 2D–2D and 3D–2D rely more on geometric constraints in the image, which are typically more stable and easier to estimate robustly.

- 3D–2D is especially strong because it uses one accurate depth estimate per correspondence, reducing noise while keeping the constraint well-conditioned.

2. Why is stereo VO better, even when only using 2D–2D?

- Stereo provides true metric scale, so the translation vector is not ambiguous.

- Depth from stereo makes feature matching more robust (epipolar lines reduce search to 1D).

- The system can reject outliers better because depth consistency can be checked.

- Scale drift is greatly reduced because stereo gives absolute depth at every frame.

Even if VO uses the 2D–2D formulation, the presence of stereo depth stabilizes the entire pipeline.

3. Ideas for improving visual odometry:

- Use stereo or multi-camera setups to recover scale and reduce drift.

- Add IMU fusion to constrain motion and smooth rotations (visual–inertial odometry).

- Use better feature detectors and descriptors (ORB, SuperPoint, etc.).

- Apply robust outlier rejection (RANSAC, M-estimators).

- Use keyframe-based bundle adjustment to refine poses.

- Incorporate loop closure to correct accumulated drift.

- Use depth priors from neural networks to stabilize scale in monocular VO.

- Use photometric optimization in addition to feature-based matching (direct VO).

- Laser

- GPS

- Compas

- IMU

# What happens over time? – Answers

1. What is the problem with accumulating transformations over time?
Small errors in each estimated motion Tk accumulate.
As we multiply more transformations together, these tiny errors grow, causing the trajectory to drift more and more from the true path. This is known as drift and is unavoidable in pure visual odometry if not corrected.

2. How can we solve it?
We reduce drift by optimizing all poses together instead of treating each motion estimate independently. This is done by re-minimizing the reprojection errors of many frames at once, correcting earlier mistakes.

3. What is bundle adjustment?
Bundle adjustment is an optimization method that jointly refines camera poses and 3D points over a window of frames. It adjusts all variables so the reprojection errors are minimized, which reduces drift and improves the overall trajectory.

# Levenberg–Marquardt in V-SLAM (short declaration)

Levenberg–Marquardt (LM) is a non-linear least-squares optimizer used in V-SLAM to refine camera poses and 3D map points. It minimizes reprojection error by blending Gauss–Newton (fast near the solution) with gradient descent (stable when far from it). LM provides robust, fast convergence during Bundle Adjustment and pose optimization, making the estimated trajectory and map significantly more accurate.